LCD firmware update Guide

You should prepare a SD card of not more than 16G (16GB or 8 GB is ok ) , and a SD card reader before the firmware update. Please format the SD card before adding firmware files to it .

The steps to format the SD card

- Step1 . Plug the SD card to the SD card reader and connect to your computer

- Step2. Then open the DOS system window : Find “Start” --> “ Run” --> input “ CMD “

- Step3. Check which disk the SD card belongs to ,and input “format/q H:/fs:fat32/a:4096 “ in the popping window.(there is a “space”char between the char “q” and “H”) (Not all SD cards are disk H: , some of them may be disk G: or disk F: , so just input the correct one)

- After inputting the command, click “ENTER”

- And then follow the guide to finish the next step.

- The steps to update the LCD

- Step1 . Unfold the file and copy the file “DWIN_SET”to the SD card.

- Make sure the file name is “DWIN_SET” and make sure the folder you copied to the SD card doesn’t include further folder .

- Step2. Power off the inverter before the SD card has been well installed.

- Step3. Plug the SD card to the holder under the LCD board ,  and make sure if it is well connected.

Step4. Power on the inverter ，and wait for the firmware update to be done.

It may take around 1 min to update the firmware

The last step is : Flash CRC32 Check: 0000,  once this step has been showed , you can wait for 5 seconds and power off the inverter and remove the SD card

Step5. Power on the inverter again and check

Note:

- If you want to refresh the LCD firmware file, just delete the old firmware file and add the new file .

- If all results are “000” , that means no file was updated successfully ,please check

- if you have formatted the SD card properly and make sure the size of the SD is not more than 16G .

- If there are more than one folder to include the firmware files, just need one folder “DWIN_SET” and open it you can see the firmware files directly . The LCD also need to find the firmware files by only open one folder.